# NetBox ↔ EDA Integration

## Technical Design and Implementation Guide

Darren Richards, AI & Cloud

October 05th, 2026

Document version: Public v1.1

Note: This is a Lab-validated integration guide. This is a summary of the full technical documentation available via the git-repository and is not official Nokia product documentation.

EDA Base Release: 26.8.1

NetBox App: v7.0.1

Table of Contents

NetBox ↔ EDA Integration	1

Technical Design and Implementation Guide	1

Introduction — EDA NetBox App	3

What the app provides	3

IPAM integration model	3

Topology integration model	4

Event-driven operation	4

Relationship to this guide	4

Executive summary	4

Document conventions	5

1. Scope and audience	5

1.1 Scope	5

1.2 Audience	6

2. Definitions and acronyms	6

Part I — Foundation	6

3. Architecture overview	6

3.1 Integration paths and namespace model	6

3.2 Namespace instantiation (onboarding prerequisites)	8

Part II — Operating Modes	9

4. Mode A — EDA-managed DCIM sync	9

4.1 Mode A — EDA-managed (topology sync on)	9

4.2 The EDAManaged tag	14

4.3 NetBox Instance CR (Mode A)	15

5. Mode B — NetBox-managed DCIM import	18

5.1 EDA namespace + instantiation (fabric-dc2)	19

5.2 NetBox prerequisites (fabric-dc2)	20

5.3 Secrets + Instance CR (fabric-dc2)	23

5.4 NetBox DCIM modelling — devices, interfaces, cables	24

5.5 ApplyTopology CR	25

5.6 Mode A vs Mode B (summary)	25

Part III — IPAM and Multi-Fabric Design	26

6. Allocations — IPAM pools	26

6.1 Architecture — one Instance per EDA namespace	26

6.2 Instance CR — full options	26

6.3 Allocation CR — all pool types	27

6.4 Lab examples — all pool types (fabric-dc2)	31

7. Decoupling topology from allocations	33

7.1 Two paths, one namespace	33

7.1.1 Bootstrap allocation pools vs NetBox-backed pools	34

7.2 Combined deployment patterns	34

7.3 What is shared vs isolated	35

7.4 Recommended build order	35

8. Multi-namespace conventions and permissions	36

8.1 Webhook URLs and tag naming	36

8.2 API token and webhook secrets (lab convention)	36

8.3 NetBox API permissions	37

Part IV — Operations and Constraints	37

9. EDA transactions	37

9.1 Per-resource operation types	37

9.2 Rule of thumb (NetBox-imported fabric)	38

9.3 Transaction vs topology import	38

9.4 Execution and inspection	38

10. Constraints and anti-patterns	38

References	39

## Introduction — EDA NetBox App

The EDA NetBox app (netbox.eda.nokia.com, built-in EDA Store catalog) integrates Nokia Event-Driven Automation with NetBox so IPAM and DCIM data can drive fabric automation while NetBox remains the inventory system of record. Per the official EDA NetBox app documentation, the app synchronizes resources between the two systems rather than replacing either platform’s native model.

### What the app provides

The NetBox app exposes four primary custom resources (API group netbox.eda.nokia.com/v1alpha1). Instance and Allocation must live in the same Kubernetes namespace (not eda-system).

| Resource | Purpose |
| --- | --- |
| Instance | Target NetBox connection (URL, API token secret, webhook signature secret); optional topology mirror when spec.sync.enabled is true |
| Allocation | Maps tagged NetBox IPAM objects to named EDA allocation pools |
| ApplyTopology | Workflow CR: import a NetBox Site (Devices and Cables) into EDA as TopoNode / TopoLink objects |
| ApplyAllocation | Workflow CR: on-demand reconcile of a single Allocation (for example after bulk NetBox edits or when webhooks are disabled) |

The app depends on the EDA Topology app (topologies.eda.nokia.com); it is installed from the EDA Store or via an AppInstaller CR.

### DCIM - Topology integration model

Two complementary paths exist:

- Topology sync (EDA → NetBox): With Instance.spec.sync.enabled: true, EDA pushes TopoNode and TopoLink objects to NetBox as Devices and Cables. Synced objects are tagged EDAManaged; spec.sync.region and spec.sync.tenant scope the mirrored Site.

- ApplyTopology (NetBox → EDA): Imports an existing NetBox Site into EDA for bootstrap when NetBox is the DCIM source of truth. Only devices whose NetBox Platform is srl become TopoNode objects; other devices may still appear as cable endpoints.

### IPAM integration model

EDA continues to allocate from its own pool CRs (IPAllocationPool, SubnetAllocationPool, IndexAllocationPool, and related types). The NetBox app creates those pools dynamically from NetBox source objects and posts allocated values back to NetBox:

| NetBox source object | NetBox status | Allocation.spec.type | EDA pool created | Typical fabric use |
| --- | --- | --- | --- | --- |
| IPAM Prefix | Active | ip-address | IPAllocationPool | System IP (host address) |
| IPAM Prefix | Active | ip-in-subnet | IPInSubnetAllocationPool | Management IP (address + mask) |
| IPAM Prefix | Container | subnet (+ subnetLength) | SubnetAllocationPool | Inter-switch link subnets |
| ASN Range | — | asn | IndexAllocationPool | BGP ASN |
| VLAN Group | — | vlan | IndexAllocationPool | VLAN ID |

When more than one NetBox prefix, ASN range, or VLAN group should feed the same pool, assign a distinct tag in NetBox and reference that tag in Allocation.spec.tags. The Allocation resource name becomes the EDA allocation pool name.

### Event-driven operation

NetBox Event Rules trigger a Webhook on create, update, or delete of IPAM and DCIM objects EDA cares about (Prefixes, VLANs, ASNs, Devices, Cables, Sites, and related types). The webhook URL is namespace- and instance-scoped:

https://<eda-host>:<port>/core/httpproxy/v1/netbox/webhook/<namespace>/<instance-name>

NetBox must also expose an API token (write-capable for IPAM; DCIM write when topology sync or ApplyTopology is used). The controller automatically creates the EDAManaged tag and matching eda_managed custom field in NetBox—do not rename or remove them.

Note: Webhook connectivity is not mandatory. It is optional, if dynamic update of NetBox is not preferred, use workflows (within EDA).

### Relationship to this guide

Product documentation covers installation, CR schemas, supported objects, and an end-to-end fabric example. This document applies that model to a multi-namespace lab: Mode A (EDA-managed topology sync) and Mode B (NetBox-managed import via ApplyTopology), plus allocation-only paths, webhook isolation, validation scripts, and constraints validated on clab-3-tier-leaf-spine-dcgw and fabric-dc2 namespaces within EDA. Further details can be found in the associated git-repository.

## Executive summary

Nokia Event-Driven Automation (EDA) integrates with NetBox through the EDA NetBox application, supporting two topology operating models and a decoupled IPAM allocation path:

| Model | Direction | sync.enabled | Primary use |
| --- | --- | --- | --- |
| Mode A — EDA-managed | EDA → NetBox DCIM mirror | true | Containerlab / live fabric; NetBox reflects deployed topology |
| Mode B — NetBox-managed | NetBox → EDA via ApplyTopology | false | Planned / greenfield design; NetBox is DCIM source of truth |
| Allocations (either mode) | NetBox tagged pools → EDA pools → consumed values back to NetBox | n/a | VLAN, ASN, system IP, management IP, ISL subnets (IPv4 /31, IPv6 /127) |

Design rule: one EDA namespace = one fabric context = one Instance CR + one dedicated webhook URL.

This guide covers architecture, prerequisites, step-by-step procedures, Kubernetes manifests, NetBox Django scripts, and lab-validated constraints (including known gaps such as Mode B ASN allocation).

Allocations support both webhook and workflow models.

## Document conventions

| Convention | Meaning |
| --- | --- |
| §N / §N.M | Section references (e.g. §5.2 = NetBox prerequisites for Mode B) |
| Mode A / Mode B | EDA-managed sync vs NetBox-managed import — see §4 and §5 |
| EDAManaged | NetBox tag marking objects created or claimed by EDA |
| kubectl | Run from WSL against the lab cluster unless noted |
| Scripts | ../scripts/ relative to this repo root |
| Manifests | ../manifests/ — Kubernetes YAML and secret templates |
| Pass criteria | Explicit checks after each procedure block |

Section numbering: §4 = Mode A · §5 = Mode B (step number = section number in §5.1 build order) · §6 = IPAM Allocations.

## 1. Scope and audience

### 1.1 Scope

In scope:

- EDA NetBox app: Instance, Allocation, ApplyTopology, ApplyAllocation

- DCIM topology (Site, Device, Interface, Cable) and IPAM pools (Prefix, VLAN Group, ASN Range)

- SR Linux (platform: srl) devices as TopoNode objects

- Lab cluster: clab-srl-leaf-spine-dcgw (EDA-managed) and fabric-dc2 (NetBox-managed pattern)

Out of scope:

- NetBox installation (Helm chart, database, ingress); the only NetBox setting this integration needs is ENFORCE_GLOBAL_UNIQUE=false

- Non-SRL platforms as TopoNode (may appear as cable endpoints only)

- EDA application development beyond the NetBox app

### 1.2 Audience

| Role | Primary sections |
| --- | --- |
| Network architect | 3, 4, 5, 7 |
| EDA operator | 5, 8, 9, 10 |
| NetBox administrator | 5, 8 |
| Automation engineer | 5, 9, 10 |

## 2. Definitions and acronyms

| Term | Definition |
| --- | --- |
| DCIM | NetBox Data Center Infrastructure Management (sites, devices, cables) |
| IPAM | NetBox IP Address Management (prefixes, VLANs, ASNs) |
| CR | Kubernetes Custom Resource in EDA |
| Instance | netbox.eda.nokia.com/v1alpha1 CR — connection to one NetBox server per EDA namespace |
| Allocation | CR mapping tagged NetBox IPAM objects to EDA allocation pools |
| ApplyTopology | Workflow CR importing a NetBox site into EDA as TopoNode/TopoLink |
| EDAManaged | Reserved NetBox tag (+ eda_managed field) marking objects EDA created or claimed |
| TopoNode | EDA topology CR representing an SR Linux node |
| Reconcile | ApplyTopology default — desired state = NetBox site only; orphans deleted |
| Namespace instantiation | CRs labeled eda.nokia.com/bootstrap=true form the template onboarding kit; EDA copies them into new namespaces via instantiation |

# Part I — Foundation

## 3. Architecture overview

### 3.1 Integration paths and namespace model

Nokia EDA’s NetBox app exposes two independent integration paths:

| Path | NetBox module | Primary direction | Trigger |
| --- | --- | --- | --- |
| Topology / inventory | DCIM | Mode-dependent | ApplyTopology + optional sync.enabled |
| Allocations | IPAM | NetBox pools → EDA → consumed values back to NetBox | Allocation + webhook / ApplyAllocation |

| Concept | Where it lives | Notes |
| --- | --- | --- |
| K8s namespace | EDA | Created first; all CRs (Instance, TopoNode, Allocation, …) live here |
| NetBox site | NetBox | Maps to the EDA namespace — naming convention ties them together. Also appears as a label on imported TopoNodes (site=fabric-dc2); the label is metadata, not the namespace itself |
| Node profile | EDA NodeProfile + NetBox device tag | eda.nokia.com/node-profile=<name> |

Site ↔ namespace mapping (lab convention):

| Mode | Who owns topology | EDA namespace | NetBox site | How they align |
| --- | --- | --- | --- | --- |
| Mode A — EDA-managed | EDA (Containerlab / live fabric) | e.g. clab-3-tier-leaf-spine-dcgw | Created by sync — site name typically matches the namespace | Instance.spec.sync pushes DCIM; NetBox Site is populated from EDA with region / tenant from the Instance |
| Mode B — NetBox-managed | NetBox (design source) | Created to match site name, e.g. fabric-dc2 | You create Site fabric-dc2 in NetBox first | ApplyTopology.spec.siteName imports that site into the same-named EDA namespace |

Namespace is defined in EDA, not NetBox. NetBox holds sites and devices; EDA decides target namespace via Instance placement and ApplyTopology (Mode B) in that namespace. The site=<name> device tag and TopoNode label mirror the NetBox site name for import filtering — they do not replace the K8s namespace.

| Mode | Example namespace | Direction |
| --- | --- | --- |
| A — EDA-managed | clab-srl-leaf-spine-dcgw | sync.enabled=true → NetBox DCIM mirror (EDAManaged) |
| B — NetBox-managed | fabric-dc2 | NetBox design source ← ApplyTopology import (sync.enabled=false) |

### 3.2 Namespace instantiation (onboarding prerequisites)

Every new fabric namespace needs relevant onboarding CRs before TopoNodes can onboard: NodeProfile, NodeUser, Init, mgmt/system IP pools, and related supporting resources. You do not need to hand-author these if namespace instantiation works.

#### How it works

At EDA installation, the default user namespace (typically eda) is populated with CRs that carry the label eda.nokia.com/bootstrap: "true" on each resource — NodeProfiles, NodeUsers, Init, allocation pools, etc. These are the template onboarding objects.

Recommended — EDA Namespace CR (same as EDA UI → Create namespace):

apiVersion: core.eda.nokia.com/v1
kind: Namespace
metadata:
  name: fabric-dc2
  namespace: eda-system          # CR object lives in eda-system
spec:
  bootstrap:
    fromNamespace: eda           # copy onboarding kit from template namespace

EDA creates the Kubernetes namespace fabric-dc2 and copies relevant onboarding CRs in one step. Use from Namespace: clab-3-tier-leaf-spine-dcgw (or another fabric) to inherit that namespace’s NodeProfile set instead of the more generic eda profiles if required. This is driven via label allocation to specific resources.

| What typically gets copied | Purpose |
| --- | --- |
| NodeProfile | OS/version, onboarding creds, mgmt pool — name goes in NetBox eda.nokia.com/node-profile= tag |
| NodeUser | Permanent management user; referenced by NodeProfile |
| NodeGroup / NodeUserGroup | Group bindings for NodeUser |
| Init | Day-0 bootstrap for selected TopoNodes |
| IPAllocationPool / SubnetAllocationPool | Mgmt and system IP pools for onboarding |

This is independent of NetBox — namespace instantiation is pure EDA onboarding. It does not set sync.enabled or NetBox integration; that comes later via the Instance CR.

#### Alternative Namespace creation (edactl CLI)

Same result as the EDA Namespace CR — run via eda-toolbox:

kubectl exec -n eda-system deploy/eda-toolbox -- \
  edactl namespace bootstrap create --from-namespace eda fabric-dc2

kubectl exec -n eda-system deploy/eda-toolbox -- \
  edactl -n fabric-dc2 namespace bootstrap list

kubectl get nodeprofiles,nodeusers,inits,ipallocationpools,subnetallocationpools -n fabric-dc2

Use --from-namespace clab-3-tier-leaf-spine-dcgw to match clab NodeProfile names.

| Approach | When to use |
| --- | --- |
| EDA Namespace CR | Recommended — EDA UI or kubectl apply; creates namespace + copies onboarding kit in one step |
| edactl namespace bootstrap create | CLI equivalent when the namespace already exists but instantiation was never run |
| Manual YAML | Custom NodeProfiles / onboarding kit only — apply NodeProfile, NodeUser, Init, pools yourself |

After instantiation — NetBox-specific adjustments:

- Note the NodeProfile name copied into the namespace (e.g. srl-leaf-spine-dcgw-srlinux-26.3.1) — set matching tag on NetBox devices.

- Patch NodeUser nodeSelector if imported TopoNodes use labels different from the clab template (e.g. add eda.nokia.com/source=netbox). Only if the NodeUser does not have an empty nodeSelector or you already built the NodeUser with relevant netbox-labels.

- Patch Init nodeSelectors similarly if nodes will onboard via ZTP.

- Namespace instantiation does not create NetBox specific CR’s (Instance / Allocation CRs) — add those separately.

The eda.nokia.com/bootstrap=true label marks which CRs belong to the onboarding CR’s. Playground / clab namespaces receive the relevant CR’s from the eda-kpt-playground package at install time; new fabric namespaces get the same core set via namespace instantiation (EDA Namespace CR or edactl namespace bootstrap create).

# Part II — Operating Modes

## 4. Mode A — EDA-managed DCIM sync

Mode A: EDA owns topology (TopoNode / TopoLink); NetBox mirrors DCIM with EDAManaged.

### 4.1 Mode A — EDA-managed (topology sync on)

Pattern: EDA owns the live fabric; NetBox is a mirror for visibility and IPAM bookkeeping.

| EDA (source of truth) | Sync | NetBox (mirror) |
| --- | --- | --- |
| Namespace e.g. clab-srl-leaf-spine-dcgw | sync.enabled=true | Site, Devices, Cables |
| TopoNode / TopoLink / Interface | → push | Tagged EDAManaged |

| Setting | Value |
| --- | --- |
| Instance.spec.sync.enabled | True |
| Instance.spec.sync.region | NetBox Region for synced Site |
| Instance.spec.sync.tenant | NetBox Tenant for synced Site |
| NetBox Site | Created/updated by EDA — not your design source |
| ApplyTopology | Usually not used (topology born in EDA / Containerlab) |

What EDA pushes to NetBox (when sync runs):

| EDA object | NetBox object | Notes |
| --- | --- | --- |
| TopoNode | Device (+ DeviceType) | Under synced Site |
| TopoLink | Cable | Between device interfaces |
| (implicit) | Site | Named from namespace / sync config |
| Interfaces | Device interfaces | Created as part of device sync |

All pushed DCIM objects receive the EDAManaged tag and EDA custom fields. Only EDA should mutate EDAManaged DCIM objects.

Typical lab example: Containerlab fabric in clab-3-tier-leaf-spine-dcgw with sync.enabled: true, region region-1, tenant tenant-a. NetBox site mirrors the clab topology; devices carry EDAManaged.

Direction of truth: EDA → NetBox for devices/cables. Changes made manually in NetBox to EDAManaged devices risk being overwritten on the next sync cycle.

Important: With sync.enabled: true, adding a Device manually in NetBox does not create a TopoNode in EDA (lab: srl-leaf-9 stayed NetBox-only). Sync is EDA → NetBox for DCIM inventory.

#### 4.1.1 Mode A — NetBox catalog prerequisites (before sync)

Before enabling Instance.spec.sync.enabled, seed NetBox with the DCIM catalog objects EDA needs to materialize TopoNode/TopoLink records. You are not creating individual device inventory here — EDA creates Sites, Devices, Cables, and (often) DeviceTypes during sync. You are ensuring lookup tables exist so the first reconcile succeeds.

| NetBox object | Required before sync? | Who creates device records? | Notes |
| --- | --- | --- | --- |
| Region | Yes — must match Instance.spec.sync.region | EDA creates Site under this region | See tenancy table below |
| Tenant | Yes — must match Instance.spec.sync.tenant | EDA assigns to synced Site/Devices | See tenancy table below |
| Manufacturer | Yes (recommended) | — | Lab: Nokia |
| DeviceType | Yes — model must match TopoNode.spec.platform | EDA may also create DeviceTypes on push; pre-seeding avoids mapping errors | Lab: 7220 IXR-D2L, 7220 IXR-D3L, 7220 IXR-D4, 7750 SR-1, 7250 IXR-X1B, 7250 IXR-X3B |
| Platform | Recommended | EDA sets platform on synced devices when mapped | Lab: srl, sros — see nb-fix-platforms.py post-sync |
| DeviceRole | Recommended | EDA maps from eda.nokia.com/role label on each TopoNode (e.g. leaf, spine, dcgw, border-leaf) — these are fabric role labels on nodes in that namespace, not separate NetBox sites. Pre-create matching DeviceRole records so sync can set Device.role. |  |
| Site / Device / Cable | No — created by sync | EDA | Tagged EDAManaged after push |
| EDAManaged tag + eda_managed field | Pre-create tag optional; CF usually EDA-created | EDA controller (or you pre-create tag) | must exist in NetBox before EDA can tag objects |
| API token | Yes — DCIM write for Mode A | — | Same token can be reused across namespaces. |
| Webhook | Yes — one per EDA namespace | — | URL path includes namespace + Instance name. |
| Event rule | Yes — bound to that webhook | — | Required, not optional. A webhook without an event rule does nothing. |

Lab-validated: Instance.status.reachable: true only proves the API token works. It does not prove DCIM sync is complete. If the event rule is missing, initial device push may partially succeed on TopoNode reconcile, but interfaces and cables will not sync. Configure webhook and event rule in NetBox before expecting a full mirror.

Disable or delete webhooks/event rules for namespaces you no longer use.

Region / Tenant per fabric — one pair per namespace with sync.enabled: true. Baseline: for example, first Mode A fabric uses region-1 / tenant-a; each additional fabric increments the number and tenant letter (region-2 / tenant-b, region-3 / tenant-c, …). Seed all pairs in the catalog script before enabling each NetBox Instance.

| Order | EDA namespace (lab) | Instance.spec.sync | Notes |
| --- | --- | --- | --- |
| Baseline | clab-3-tier-leaf-spine-dcgw | region: region-1, tenant: tenant-a | First Mode A fabric |
| Second | clab-srl-leaf-spine-dcgw | region: region-2, tenant: tenant-b | Second Mode A fabric |
| Planned import | fabric-dc2 | sync.enabled: false | Mode B — assign Region/Tenant on NetBox Site manually |

Ensure the EDAManaged tag exists in NetBox (pre-create or let EDA create on first reconcile with Extras Tag/CF permissions) before expecting tagged sync results.

Roles (leaf / spine / dcgw): In EDA, each TopoNode carries labels such as eda.nokia.com/role=leaf and site: <fabric-name>. The fabric is the EDA namespace (and synced NetBox Site). Role labels describe the node’s function in that fabric. The catalog seed pre-creates NetBox DeviceRole entries with the same names so Mode A sync can populate Device.role.

Mode B: you set equivalent tags on NetBox devices before ApplyTopology.

Lab fabric platforms (match TopoNode.spec.platform in clab namespaces):

| TopoNode.spec.platform | u_height | OS (typical) |
| --- | --- | --- |
| 7220 IXR-D2L | 1 | SR Linux |
| 7220 IXR-D3L | 1 | SR Linux |
| 7220 IXR-D4 | 1 | SR Linux |
| 7750 SR-1 | 2 | SR OS |
| 7250 IXR-X1B | 1 | SR Linux |
| 7250 IXR-X3B | 1 | SR Linux |

Adding more hardware: The seed script loads lab + extended Nokia SKUs by default (DEVICE_TYPES = LAB_DEVICE_TYPES + EXTENDED_DEVICE_TYPES). Trim to LAB_DEVICE_TYPES only if you want a minimal catalog. Every distinct TopoNode.spec.platform needs a matching DeviceType.model (exact string). Re-run the script — get_or_create is idempotent and updates u_height for example if changed.

Extended Nokia catalog (EXTENDED_DEVICE_TYPES): Merged into the default seed list below. u_height from Nokia hardware datasheets. Lab models (D2L/D3L/D4, SR-1, X1B/X3B) are in LAB_DEVICE_TYPES; extended adds the rest.

| Family | DeviceType.model | u_height | Notes |
| --- | --- | --- | --- |
| 7215 IXS | 7215 IXS-A1 | 1 |  |
| 7220 IXR-D | 7220 IXR-D1 | 1 | D2L, D3L, D4 in lab table |
|  | 7220 IXR-D5 | 1 |  |
| 7220 IXR-H | 7220 IXR-H2 | 4 |  |
|  | 7220 IXR-H3 | 1 |  |
|  | 7220 IXR-H4-32D | 1 |  |
|  | 7220 IXR-H4 | 2 |  |
|  | 7220 IXR-H5-32D | 1 |  |
|  | 7220 IXR-H5-64D | 2 |  |
|  | 7220 IXR-H5-64O | 2 | OSFP112 variant |
|  | 7220 IXR-H6-64 | 3 |  |
| 7250 IXR-X | 7250 IXR-X4 | 1 | X1B/X3B in lab table |
| 7250 IXR-e | 7250 IXR-6e | 10 |  |
|  | 7250 IXR-10e | 16 |  |
|  | 7250 IXR-18e | 35 |  |
| 7750 SR | 7750 SR-7 | 8 | SR-1 in lab table |
|  | 7750 SR-12 | 14 |  |
|  | 7750 SR-12e | 22 |  |
| 7750 SR-1x | 7750 SR-1x-48D | 2 | 48D family |
|  | 7750 SR-1-48D | 2 |  |
|  | 7750 SR-1-24D | 2 |  |
|  | 7750 SR-1x-92S | 2 | 92S family |
|  | 7750 SR-1-92S | 2 |  |
|  | 7750 SR-1-46S | 2 |  |
| 7750 SR-s | 7750 SR-1s | 3 |  |
|  | 7750 SR-1se | 3 |  |
|  | 7750 SR-2s | 5 |  |
|  | 7750 SR-2se | 5 |  |
|  | 7750 SR-7s | 17 | Datasheet: 16 or 17 RU depending on power config |
|  | 7750 SR-14s | 28 | Datasheet: 27 or 28 RU depending on power config |

Platform string matching: DeviceType.model must match TopoNode.spec.platform exactly (case/spacing). Lab uses 7250 IXR-X1B and 7250 IXR-X3B. Check platforms with:
kubectl get toponodes -n <ns> -o jsonpath='{range .items[*]}{.spec.platform}{"\n"}{end}' | sort -u
Interface port templates (QSFP-DD counts, etc.) are not created by the catalog script. Scripts available via git repository.

Build order (Mode A):

- NetBox: Region, Tenant, Manufacturer, DeviceTypes, Platforms, DeviceRoles (catalog seed — check Git Repository referenced later)

- NetBox: API token (DCIM write), webhook + event rule for this namespace— before expecting full sync

- EDA: namespace exists; Containerlab / workflows have TopoNode/TopoLink CRs

- EDA: Instance CR with sync.enabled: true, matching region/tenant → verify reachable=true

- Wait for sync — Devices, interfaces, Site, Cables appear in NetBox with EDAManaged

- Optional post-sync fixes: Changes to platform assignment, device-type u_height etc (run nb-fix-platforms.py)

### 4.2 The EDAManaged tag

EDAManaged is a reserved NetBox tag (and matching boolean custom fields) used by the Nokia EDA NetBox app to mark objects EDA created or claimed.

Important: The tag is not defined on the EDA Instance CR. It is a NetBox object that must exist (or be created by EDA) before EDA can tag synced or allocated objects. Mode B design devices must not carry this tag — see §5.

#### Who creates EDAManaged / eda managed CF’s

| Object | Typical creator | When |
| --- | --- | --- |
| EDAManaged tag | EDA NetBox controller on first reconcile, or you pre-create in NetBox | Before/at first sync or first allocation write-back |
| Eda custom fields | EDA controller on first reconcile (recommended) | Same — requires Extras → Custom Field API permission (needs checking) |

EDA needs Extras → Tag and Extras → Custom Field write permission on the API token to auto-create these. If reconcile fails to tag objects, verify permissions and that the EDAManaged tag exists in NetBox (Customization → Tags).

Pre-create (optional but valid): Create tag EDAManaged (slug edamanaged) in NetBox before applying the Instance CR if you want the tag visible before first reconcile. Do not rename or delete after EDA uses it. The eda_managed custom field is best left for EDA to create on first reconcile.

#### What it marks

Objects NetBox should treat as owned by EDA:

| Object | When it gets EDAManaged |
| --- | --- |
| Site, Device, Cable, DeviceType (sync path) | EDA pushes them via topology sync (sync.enabled: true) |
| Your design site/devices (Mode B) | Never — you create these without EDAManaged; EDA imports them to TopoNodes |

#### What “only EDA should mutate” means

- Do not manually edit EDAManaged devices, cables, or sites in the NetBox UI for routine changes — EDA is the source of truth for those objects (Mode A) or for tracking allocation ownership (IPAM).

- On the next sync cycle, EDA may overwrite manual NetBox edits to EDAManaged DCIM objects.

- Do not delete the EDAManaged tag or eda_managed custom field — the NetBox app depends on them.

- Do not run ApplyTopology against a site or devices that already carry EDAManaged — the workflow expects a NetBox-owned design source (Mode B only).

#### How to use it in practice

| Task | Action |
| --- | --- |
| See what EDA synced from clab | NetBox → Devices → filter tag EDAManaged |
| See what EDA allocated from pools | NetBox → IP Addresses / VLANs → filter EDAManaged |
| Your planned fabric (Mode B) | Site and devices without EDAManaged; only consumed IPs/VLANs get it after allocation |
| Troubleshoot “who owns this device?” | If EDAManaged → EDA; if not → your team (or not yet synced) |

### 4.3 NetBox Instance CR (Mode A)

The NetBox Instance CR connects EDA to NetBox for this namespace. For Mode A, set sync.enabled: true with matching region / tenant.

After you configure NetBox webhook and verify sync. Site and devices are created by EDA on first sync — not pre-modelled in NetBox.

Allocation pool tags and Allocation CRs are in §6 (orthogonal to DCIM).

Region / Tenant convention: Lab baseline namespace clab-3-tier-leaf-spine-dcgw → region-1 / tenant-a. Each additional Mode A namespace increments: region-2 / tenant-b, region-3 / tenant-c, … in this implementation example.

Baseline fabric: clab-3-tier-leaf-spine-dcgw. Apply secrets YAML first, then Instance YAML — same EDA namespace.

Credentials — edit stringData in the secrets file before apply:

| Value | NetBox UI location | YAML key |
| --- | --- | --- |
| API token | Admin → Authentication → API Tokens | stringData.apiToken on Secret netbox-api-token |
| Webhook secret | Operations → Integrations → Webhooks → Secret (this namespace) | stringData.signatureKey on Secret netbox-webhook-signature |

Lab: one API token across namespaces is possible; webhook secret unique per namespace.

#### Mode A baseline — clab-3-tier-leaf-spine-dcgw (region-1 / tenant-a)

Save each block below as the named file, edit stringData, then apply in order.

##### secrets-clab-3-tier-leaf-spine-dcgw.yaml

# Edit stringData from NetBox UI before apply.
# apiToken     — Admin → Authentication → API Tokens
# signatureKey — Operations → Integrations → Webhooks → Secret (this namespace)
apiVersion: v1
kind: Secret
metadata:
  name: netbox-webhook-signature
  namespace: clab-3-tier-leaf-spine-dcgw
type: Opaque
stringData:
  signatureKey: paste-webhook-secret-from-netbox
---
apiVersion: v1
kind: Secret
metadata:
  name: netbox-api-token
  namespace: clab-3-tier-leaf-spine-dcgw
type: Opaque
stringData:
  apiToken: paste-netbox-api-token

##### instance-clab-3-tier-leaf-spine-dcgw.yaml

# Baseline Mode A — region-1 / tenant-a. Apply secrets YAML first.
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Instance
metadata:
  name: netbox
  namespace: clab-3-tier-leaf-spine-dcgw
spec:
  url: http://netbox.netbox.svc.cluster.local:80
  apiToken: netbox-api-token
  signatureKey: netbox-webhook-signature
  sync:
    enabled: true
    region: region-1
    tenant: tenant-a

kubectl apply -f secrets-clab-3-tier-leaf-spine-dcgw.yaml
kubectl apply -f instance-clab-3-tier-leaf-spine-dcgw.yaml
kubectl get instance.netbox.eda.nokia.com netbox -n clab-3-tier-leaf-spine-dcgw -o jsonpath='reachable={.status.reachable}{"\n"}'

Pass: reachable=true

#### Mode A second fabric — clab-srl-leaf-spine-dcgw (region-2 / tenant-b) – just as reference for multi-tenant alignment with NB.

##### secrets-clab-srl-leaf-spine-dcgw.yaml

# Second fabric — unique webhook secret for this namespace.
apiVersion: v1
kind: Secret
metadata:
  name: netbox-webhook-signature
  namespace: clab-srl-leaf-spine-dcgw
type: Opaque
stringData:
  signatureKey: paste-webhook-secret-from-netbox
---
apiVersion: v1
kind: Secret
metadata:
  name: netbox-api-token
  namespace: clab-srl-leaf-spine-dcgw
type: Opaque
stringData:
  apiToken: paste-netbox-api-token

##### instance-clab-srl-leaf-spine-dcgw.yaml

# Second Mode A fabric — region-2 / tenant-b. Apply secrets YAML first.
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Instance
metadata:
  name: netbox
  namespace: clab-srl-leaf-spine-dcgw
spec:
  url: http://netbox.netbox.svc.cluster.local:80
  apiToken: netbox-api-token
  signatureKey: netbox-webhook-signature
  sync:
    enabled: true
    region: region-2
    tenant: tenant-b

kubectl apply -f secrets-clab-srl-leaf-spine-dcgw.yaml
kubectl apply -f instance-clab-srl-leaf-spine-dcgw.yaml
kubectl get instance.netbox.eda.nokia.com netbox -n clab-srl-leaf-spine-dcgw -o jsonpath='reachable={.status.reachable}{"\n"}'

| NetBox Instance field | Purpose |
| --- | --- |
| spec.url | NetBox API base URL |
| spec.apiToken | Kubernetes Secret with NetBox REST token |
| spec.signatureKey | Kubernetes Secret with webhook HMAC secret |
| spec.sync.enabled | true for Mode A (EDA → NetBox push) |
| spec.sync.region / .tenant | NetBox Region and Tenant for synced Site |
| spec.disableWebhook | true disables auto-reconcile on NetBox events |

Status: status.reachable, status.errorReason, status.lastChecked — reachable: true before sync.

#### Mode A checklist (before first sync)

| # | NetBox | EDA |
| --- | --- | --- |
| 1 | Region + Tenant exist | TopoNodes in namespace |
| 2 | Catalog seeded — DeviceTypes, Roles | Namespace instantiated |
| 3 | API token + webhook + event rule — both required | Secrets applied |
| 4 | EDAManaged pre-created or token has Extras Tag/CF write | Instance CR applied; reachable=true |
| 5 | — | Wait for sync → Site + devices + interfaces + cables with EDAManaged |

## 5. Mode B — NetBox-managed DCIM import

NetBox is the design source for DCIM. EDA imports topology via ApplyTopology. IPAM allocation pools are optional.

Lab example: EDA namespace fabric-dc2, NetBox site fabric-dc2, sync.enabled: false.

| From (NetBox) | To (EDA) | Mechanism |
| --- | --- | --- |
| DCIM site fabric-dc2 (no EDAManaged) | Namespace fabric-dc2 | ApplyTopology |
| Devices, interfaces, cables | TopoNode, Interface, TopoLink | Import only |

| Setting | Value |
| --- | --- |
| Instance.spec.sync.enabled | false |
| NetBox Site | You create — must not be EDAManaged |
| ApplyTopology | Required — workflow CR reads NetBox site → TopoNode / TopoLink |
| Device tags | eda.nokia.com/node-profile=... (required), role, custom labels |

NetBox → EDA mapping:

| NetBox (DCIM) | EDA CR | Requirements |
| --- | --- | --- |
| Device | TopoNode | Platform = srl; eda.nokia.com/node-profile=<NodeProfile> tag |
| Interface | Interface | On SRL devices |
| Cable | TopoLink | Both ends resolved |
| Site name | TopoNode label site: <siteName> | Not an EDA namespace |

ApplyTopology default: operation: Reconcile — orphans in the EDA namespace are deleted. Use a dedicated namespace; never import into a clab/Mode A namespace.

Build order — section number = step number:

| Step | Action |
| --- | --- |
| 1 | EDA namespace + instantiation; NodeUser patch |
| 2 | NetBox prerequisites (region, tenant, site, webhook, event rule) — before EDA secrets |
| 3 | secrets-fabric-dc2.yaml + instance-fabric-dc2.yaml → reachable=true |
| 4 | NetBox DCIM — nb-test-fabric-dc2.py (devices, interfaces, cables) |
| 5 | applytopology-fabric-dc2.yaml → TopoNodes / TopoLinks |
| 6 | (Optional) NetBox IPAM allocation pools + allocations-fabric-dc2.yaml |

### 5.1 EDA namespace + instantiation (fabric-dc2)

Create a dedicated fabric namespace before any NetBox Instance or ApplyTopology.

File: eda-namespace-fabric-dc2.yaml (same as EDA UI → Create namespace)

# Mode B — recommended. Matches EDA UI "Create namespace".
apiVersion: core.eda.nokia.com/v1
kind: Namespace
metadata:
  name: fabric-dc2
  namespace: eda-system          # CR object is stored in eda-system
spec:
  bootstrap:
    fromNamespace: eda           # or clab-3-tier-leaf-spine-dcgw | clab-srl-leaf-spine-dcgw

| Onboarding object | Purpose for Mode B |
| --- | --- |
| NodeProfile | Name goes in NetBox device tag eda.nokia.com/node-profile=<name> |
| NodeUser / Init | Onboarding when TopoNodes are deployed (patch below for NetBox-sourced nodes) |
| IPAllocationPool / SubnetAllocationPool | Mgmt and system IP pools for future node onboarding |

Patch NodeUser so imported TopoNodes (label eda.nokia.com/source=netbox from your NetBox device tags) match SSH/credential bindings:

kubectl patch nodeuser admin -n fabric-dc2 --type=json -p='[
  {"op":"add","path":"/spec/groupBindings/-","value":{
    "groups":["sudo"],
    "nodeSelector":["eda.nokia.com/source=netbox"]
  }}
]'

Pass: kubectl get nodeprofiles -n fabric-dc2 returns at least one profile; NODE_PROFILE printed for use

### 5.2 NetBox prerequisites (fabric-dc2)

#### 5.2.1 Catalog objects (verify — usually already seeded)

If you already ran nb-seed-eda-catalog.py, these already exist in NetBox. Confirm in the UI; re-run the seed script if anything is missing or you need to add new items.

| Object | NetBox UI path | Lab value | Needed for Mode B |
| --- | --- | --- | --- |
| Manufacturer | DCIM → Manufacturers | Nokia | Yes |
| Device type | DCIM → Device Types | 7220 IXR-D3L, 7220 IXR-D4 | Yes — device script looks up types |
| Platform | DCIM → Platforms | srl | Yes |
| Device role | DCIM → Device Roles | leaf, spine | Yes |
| Region | Tenancy → Regions | region-3 | Yes — assign to site |
| Tenant | Tenancy → Tenants | tenant-c | Yes |
| API token | Admin → API Tokens → Add | One token (reuse Mode A) | Yes |

API token (if you do not already have one from Mode A):

| Field | Value |
| --- | --- |
| Description | e.g. EDA lab |
| Write enabled | Yes |
| Permissions | Grant write on DCIM (Device, Interface, Cable, Site), IPAM (Prefix, VLAN Group, ASN Range, IP Address, VLAN, ASN), and Extras (Tag, Custom Field) |

Copy the token once — it goes in secrets-fabric-dc2.yaml → apiToken.

#### 5.2.2 Create region, tenant, and site in NetBox

NetBox UI only — Tenancy and DCIM menus.

Example — Region

- Tenancy → Regions → Add

- Name: region-3

- Create

Example — Tenant

- Tenancy → Tenants → Add

- Name: tenant-c

- Create

Example — Site (must match ApplyTopology.spec.siteName)

- DCIM → Sites → Add

- Name: fabric-dc2

- Status: Planned

- Region: region-3

- Tenant: tenant-c

- Tags: leave empty — do not add EDAManaged

- Create

| # | NetBox UI path | Lab value | Notes |
| --- | --- | --- | --- |
| 1 | Tenancy → Regions → Add | Name region-3 | Skip if catalog seed already created it |
| 2 | Tenancy → Tenants → Add | Name tenant-c | Skip if catalog seed already created it |
| 3 | DCIM → Sites → Add | Name fabric-dc2 | Region region-3, Tenant tenant-c, Status planned; no EDAManaged tag |

Site name must match ApplyTopology.spec.siteName (fabric-dc2). The device script can get_or_create the site if you skip step 3 — creating it here assigns region/tenant in the UI first.

#### 5.2.3 Create webhook in NetBox

NetBox UI only — Customization → Webhooks → Add (NetBox 4.x: Operations → Integrations → Webhooks → Add)

Full example:

| Field | Value |
| --- | --- |
| Name | EDA fabric-dc2 |
| Enabled | ✓ |
| URL | https://eda-api.eda-system.svc.cluster.local:443/core/httpproxy/v1/netbox/webhook/fabric-dc2/netbox |
| HTTP method | POST |
| HTTP content type | application/json |
| Secret | e.g. fabric-dc2-wh-secret-change-me — generate a new random string; copy for secrets-fabric-dc2.yaml → signatureKey (§5.4) |
| SSL verification | Disabled (lab self-signed) |
| CA file path | (empty) |

URL must be reachable from the NetBox pod (not browser localhost):

kubectl get svc eda-api -n eda-system
# Typical in-cluster URL:
# https://eda-api.eda-system.svc.cluster.local:443/core/httpproxy/v1/netbox/webhook/fabric-dc2/netbox

Do not reuse the clab webhook URL or signing secret — each EDA namespace needs its own webhook + secret.

#### 5.2.4 Create event rule in NetBox

NetBox UI only — Customization → Event Rules → Add (NetBox 4.x: Operations → Integrations → Event Rules → Add)

Full example:

| Field | Value |
| --- | --- |
| Name | EDA fabric-dc2 events |
| Enabled | ✓ |
| Event types | Object created · Object updated · Object deleted |
| Action type | Webhook |
| Webhook | EDA fabric-dc2 |
| Object types | Enable all rows below |

| Object type (NetBox UI) | Topology | Allocations |
| --- | --- | --- |
| DCIM → Cable | Optional | — |
| DCIM → Device | Optional | — |
| DCIM → Device Type | Optional | — |
| DCIM → Site | Optional | — |
| IPAM → ASN | — | Yes |
| IPAM → ASN Range | — | Yes |
| IPAM → IP Address | — | Yes |
| IPAM → Prefix | — | Yes |
| IPAM → VLAN | — | Yes |
| IPAM → VLAN Group | — | Yes |

For the full lab (topology + optional allocations), enable all types — same object set as Mode A; only the webhook URL targets fabric-dc2.

#### 5.2.5 Not needed until later

| Item | When |
| --- | --- |
| Devices, interfaces, cables | nb-test-fabric-dc2.py |
| VLAN groups, prefixes, ASN ranges | optional |
| EDAManaged on site or devices | Never on Mode B design objects |

#### 5.2.6 Global setting (one-time, NetBox UI or admin)

Confirm ENFORCE_GLOBAL_UNIQUE=false.

Pass: Site fabric-dc2 visible in NetBox UI (no EDAManaged); webhook + event rule saved; webhook Secret and API token copied ready for §5.4.

### 5.3 Secrets + Instance CR (fabric-dc2)

#### 5.3.1 secrets-fabric-dc2.yaml

# Mode B — edit stringData from NetBox UI before apply.
apiVersion: v1
kind: Secret
metadata:
  name: netbox-webhook-signature
  namespace: fabric-dc2
type: Opaque
stringData:
  signatureKey: paste-webhook-secret-from-netbox
---
apiVersion: v1
kind: Secret
metadata:
  name: netbox-api-token
  namespace: fabric-dc2
type: Opaque
stringData:
  apiToken: paste-netbox-api-token

#### 5.3.2 instance-fabric-dc2.yaml

# Mode B — NetBox design source. sync.enabled: false
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Instance
metadata:
  name: netbox
  namespace: fabric-dc2
spec:
  url: http://netbox.netbox.svc.cluster.local:80
  apiToken: netbox-api-token
  signatureKey: netbox-webhook-signature
  sync:
    enabled: false

kubectl apply -f secrets-fabric-dc2.yaml
kubectl apply -f instance-fabric-dc2.yaml
kubectl get instance.netbox.eda.nokia.com netbox -n fabric-dc2 -o jsonpath='reachable={.status.reachable}{"\n"}'

### 5.4 NetBox DCIM modelling — devices, interfaces, cables

After §5.3 (Instance reachable). Creates leaf/spine devices in NetBox — via the script below (run inside NetBox pod) or manually in the NetBox UI.

#### 5.4.1 Topology summary

Django ORM script executed inside the NetBox pod (manage.py shell). Creates devices, interfaces, and cables at site fabric-dc2. Site may already exist from §5.3.2; script uses get_or_create. Script available in git repository.

| Role | Names | Device type | Uplinks |
| --- | --- | --- | --- |
| Leaf | leaf-dc2-01 … leaf-dc2-04 | 7220 IXR-D3L | ethernet-1/49 → spine-01, ethernet-1/50 → spine-02 |
| Spine | spine-dc2-01, spine-dc2-02 | 7220 IXR-D4 | ethernet-1/1–ethernet-1/4 (one per leaf link) |

#### 5.4.2 Manual alternative (NetBox UI)

Create each device in DCIM → Devices with platform srl, device types above, and tags:

eda.nokia.com/node-profile=<NodeProfile-from-§5.2>
eda.nokia.com/role=leaf
site=fabric-dc2

Add interfaces and cables in DCIM. Optional cable tag: eda.nokia.com/role=interSwitch. Cable type is optional in NetBox — leave blank unless you want DCIM documentation; it is not imported into EDA. Plain string tags without = are ignored by ApplyTopology except site=<siteName>.

Do not add eda.nokia.com/source=netbox to device tags. EDA applies that label internally on imported TopoNodes; NetBox device tags with that key are copied to TopoNode labels and rejected by the CE.

### 5.5 ApplyTopology CR

File: applytopology-fabric-dc2.yaml

apiVersion: netbox.eda.nokia.com/v1alpha1
kind: ApplyTopology
metadata:
  name: import-fabric-dc2
  namespace: fabric-dc2
spec:
  instanceName: netbox
  siteName: fabric-dc2
  deviceTags: []

kubectl apply -f applytopology-fabric-dc2.yaml
kubectl get applytopology import-fabric-dc2 -n fabric-dc2 -o yaml
kubectl get toponodes -n fabric-dc2

Pass: TopoNode / TopoLink count matches NetBox site (6 TopoNodes, 8 TopoLinks for this lab).

Hands-on walkthrough and references are in the full technical documentation/git repository.

### 5.6 Mode A vs Mode B (summary)

|  | Mode A | Mode B |
| --- | --- | --- |
| Source of truth | EDA TopoNode / TopoLink | NetBox Site / Devices / Cables |
| NetBox Site | EDA creates on sync | You create — not EDAManaged |
| sync.enabled | true | false |
| Import to EDA | Containerlab / workflows | ApplyTopology |
| DCIM token perms | create / update / delete | read (import only) |

| Mode | EDAManaged on DCIM source? | EDAManaged on allocated IPAM? |
| --- | --- | --- |
| A — EDA-managed (sync.enabled: true) | Yes — all synced Site/Device/Cable | Yes — when EDA hands out pool values |
| B — NetBox-managed (sync.enabled: false) | No on your design site/devices | Yes — when EDA hands out pool values |

Rule: Never run ApplyTopology against a site that EDA already owns (EDAManaged). Never enable sync.enabled on a namespace whose NetBox site is your design source.

# Part III — IPAM and Multi-Fabric Design

## 6. Allocations — IPAM pools

This section covers architecture and CR reference – we are using Mode B namespace for reference.

IPAM allocation pools are orthogonal to DCIM Mode A and Mode B . Same Instance CR; pool tags on NetBox IPAM objects.

### 6.1 Architecture — one Instance per EDA namespace

A single NetBox server can serve many EDA namespaces. Each namespace gets its own integration binding:

| NetBox (one server) | Webhook path pattern |
| --- | --- |
| DCIM sites + tagged IPAM pools | /eda/<namespace>/netbox per fabric |

| EDA namespace | Instance | TopoNodes | Allocation CRs |
| --- | --- | --- | --- |
| clab-srl-leaf-spine-dcgw | sync.enabled: true | EDA-owned (clab) | IndexAllocationPool, … |
| fabric-dc2 | sync.enabled: false | NetBox-imported | IPAllocationPool, … |

Per namespace you deploy:

- Secrets: netbox-api-token, netbox-webhook-signature (lab reuses one API token; webhook secret is always per namespace)

- Instance CR

- NetBox webhook URL: https://<EDA>:9443/core/httpproxy/v1/netbox/webhook/<NAMESPACE>/<INSTANCE_NAME>

- NetBox Event Rule (can be one rule covering all object types — webhook URL differs per namespace)

- Zero or more Allocation CRs

- Optional: ApplyTopology / ApplyAllocation workflows

Instance and Allocation must live in the same namespace. The Allocation controller resolves spec.instance by name within that namespace only.

### 6.2 Instance CR — full options

apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Instance
metadata:
  name: netbox                    # referenced by Allocation + ApplyTopology
  namespace: fabric-dc2           # one per fabric namespace
spec:
  url: http://netbox.netbox.svc.cluster.local:80
  apiToken: netbox-api-token      # secret name; key: apiToken (base64)
  signatureKey: netbox-webhook-signature  # secret name; key: signatureKey

  # Optional
  checkInterval: 1m               # NetBox reachability probe (default 1m)
  timeout: 10s                    # per-request API timeout (default 10s)
  disableWebhook: false           # true = no auto-reconcile on NetBox events
  tls:
    skipVerify: false
    trustBundle: my-ca-bundle     # ConfigMap with trust-bundle.pem

  sync:
    enabled: false                # true = EDA→NetBox topology push (Mode A)
    region: region-1              # NetBox Region for synced Site (sync only)
    tenant: tenant-a              # NetBox Tenant for synced Site (sync only)

| Field | Affects topology? | Affects allocations? |
| --- | --- | --- |
| sync.enabled | Yes — EDA→NetBox device/cable push | No |
| sync.region / sync.tenant | Yes (synced Site metadata) | No |
| disableWebhook | Yes (Mode A) – device push is controller driven, but interace and cable sync needs the webhook + event rule | Yes — use ApplyAllocation manually if true |
| url, apiToken, TLS | Both | Both |

Status fields: reachable, errorReason, lastChecked

### 6.3 Allocation CR — all pool types

Allocation maps tagged NetBox IPAM objects → EDA allocation pools. Matching is by plain string tags on IPAM objects (not key=value device tags).

apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-vlan-pool   # becomes EDA pool name
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox                # Instance CR in same namespace
  tags:
    - eda-fabric-dc2-vlan          # plain string tag on NetBox VLAN Group
  type: vlan
  # subnetLength: 31              # required for type: subnet only (ISL point-to-point /31)
  description: ""                 # optional

#### Complete type matrix

| spec.type | NetBox source | Prefix status | EDA pool CRD | Typical use |
| --- | --- | --- | --- | --- |
| ip-address | Prefix | Active | IPAllocationPool | System / loopback IPs |
| ip-in-subnet | Prefix | Active | IPInSubnetAllocationPool | Management IP (address + mask) |
| subnet | Prefix | Container | SubnetAllocationPool | ISL / point-to-point link subnets |
| asn | ASN Range | n/a | IndexAllocationPool | BGP private ASNs |
| vlan | VLAN Group | n/a | IndexAllocationPool | VLAN IDs |

#### ASN allocation and sync.enabled (lab finding)

VLAN and prefix-based pools (vlan, ip-address, ip-in-subnet, subnet) reconcile from tagged NetBox IPAM objects only — they work in both Mode A and Mode B.

ASN behaviour is different in App version 4.0.3. The eda-netbox allocation reconciler seems to require the namespace site to be in its internal synced-site cache. That cache is populated when Instance.spec.sync.enabled: true (Mode A DCIM sync). With sync.enabled: false (Mode B), even a correctly tagged ASN range in NetBox and a healthy ApplyTopology import will log:

site not yet synced ... sync.enabled=false

| Mode | sync.enabled | VLAN / IP pools | ASN pool |
| --- | --- | --- | --- |
| A — EDA-managed (clab-3-tier-leaf-spine-dcgw) | true | Matched | Matched |
| B — NetBox-managed (fabric-dc2) | false | Matched | Blocked (Under investigation) |

Sharing the same Instance name (netbox) across namespaces — each namespace resolves spec.instance locally and overlapping ASN numeric ranges is supported — matching is by tag.

In this release combination - Mode B lab: expect 4/5 Allocation CRs matched; use Mode A namespace for full ASN pool testing until Nokia documents a Mode B workaround.

Tag rule: one distinct tag per pool mapping. Multiple NetBox objects with the same tag can feed one Allocation (status shows matchedPrefixes / matched ranges).

Example — five pools in one namespace (fabric-dc2 tags per §8.1):

# --- VLAN (IndexAllocationPool) ---
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-vlan-pool
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox
  tags: [eda-fabric-dc2-vlan]
  type: vlan
---
# --- ASN (IndexAllocationPool) ---
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-asn-pool
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox
  tags: [eda-fabric-dc2-asn]
  type: asn
---
# --- System IPs (IPAllocationPool) ---
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-systemip
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox
  tags: [eda-fabric-dc2-systemip]
  type: ip-address
---
# --- Management IPs (IPInSubnetAllocationPool) ---
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-mgmt
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox
  tags: [eda-fabric-dc2-mgmt]
  type: ip-in-subnet
---
# --- ISL subnets (SubnetAllocationPool) ---
apiVersion: netbox.eda.nokia.com/v1alpha1
kind: Allocation
metadata:
  name: nb-fabric-dc2-isl
  namespace: fabric-dc2
spec:
  enabled: true
  instance: netbox
  tags: [eda-fabric-dc2-isl]
  type: subnet
  subnetLength: 31

#### NetBox IPAM setup (summary)

| spec.type | NetBox object | Tag (fabric-dc2) | Status / range |
| --- | --- | --- | --- |
| vlan | VLAN Group | eda-fabric-dc2-vlan | VID range e.g. 100–199 |
| asn | ASN Range | eda-fabric-dc2-asn | e.g. 4200000000–4200000999 |
| ip-address | Prefix | eda-fabric-dc2-systemip | Active e.g. 10.0.1.0/24 |
| ip-in-subnet | Prefix | eda-fabric-dc2-mgmt | Active e.g. 192.168.100.0/24 |
| subnet | Prefix | eda-fabric-dc2-isl | Container e.g. 10.255.0.0/16 |

Full NetBox UI steps and per-pool examples:

IPAM objects are not scoped to a DCIM site. Separation across namespaces is by tag naming, not site membership.

#### Trigger and verify

File: applyallocationrefresh-fabric-dc2.yaml (on-demand reconcile — repeat per pool name)

apiVersion: netbox.eda.nokia.com/v1alpha1
kind: ApplyAllocation
metadata:
  name: refresh-fabric-dc2-vlan
  namespace: fabric-dc2
spec:
  allocation: nb-fabric-dc2-vlan-pool

kubectl apply -f applyallocation-refresh-fabric-dc2.yaml
kubectl get instance,allocation -n fabric-dc2
kubectl get allocation nb-fabric-dc2-vlan-pool -n fabric-dc2 -o yaml
kubectl get ipallocationpools,subnetallocationpools,indexallocationpools -n fabric-dc2

When EDA consumes a value (Fabric app, onboarding, etc.):

- Individual IP / VLAN / ASN is created in NetBox IPAM

- Object gets EDAManaged tag + EDA custom fields (allocation owner, requesting CR)

- Parent pool prefix / VLAN group / ASN range stays your object — not fully taken over

### 6.4 Lab examples — all pool types (fabric-dc2)

Canonical walkthrough: This subsection is a short index; full NetBox UI steps, per-pool Allocation YAML, and allocations-fabric-dc2.yaml are in the Git Repo.

Create tagged IPAM objects in NetBox before kubectl apply -f allocations-fabric-dc2.yaml (NB allocations CR). Tags are plain strings (not key=value device tags).

#### 6.4.0 Run all pools (script)

File: nb-test-allocation-pools-fabric-dc2.py. Creates all five objects + tags in one run: Available via git repository.

SCRIPT=../scripts/nb-test-allocation-pools-fabric-dc2.py
POD=$(kubectl get pod -n netbox -l app.kubernetes.io/name=netbox -o jsonpath='{.items[0].metadata.name}')
kubectl cp "$SCRIPT" "netbox/${POD}:/tmp/nb-test-allocation-pools-fabric-dc2.py"
kubectl exec -n netbox "$POD" -- python /opt/netbox/netbox/manage.py shell -c "exec(open('/tmp/nb-test-allocation-pools-fabric-dc2.py').read())"

Then apply EDA side:

kubectl apply -f allocations-fabric-dc2.yaml
kubectl get allocation -n fabric-dc2

#### 6.4.1 VLAN (type: vlan)

| Item | Lab value |
| --- | --- |
| NetBox UI | IPAM → VLAN Groups → Add |
| Name | fabric-dc2-vlans |
| VID ranges | 100–199 |
| Tag | eda-fabric-dc2-vlan (Extras → Tags → Add if missing; assign on VLAN Group) |
| Allocation CR | nb-fabric-dc2-vlan-pool, spec.type: vlan, spec.tags: [eda-fabric-dc2-vlan] |
| EDA pool CRD | IndexAllocationPool |

#### 6.4.2 ASN (type: asn)

| Item | Lab value |
| --- | --- |
| NetBox UI | IPAM → ASN Ranges → Add |
| Name | fabric-dc2-asns |
| RIR | Private |
| Range | 4200000000 – 4200000999 |
| Tag | eda-fabric-dc2-asn |
| Allocation CR | nb-fabric-dc2-asn-pool, spec.type: asn |
| EDA pool CRD | IndexAllocationPool |

#### 6.4.3 System IP (type: ip-address)

| Item | Lab value |
| --- | --- |
| NetBox UI | IPAM → Prefixes → Add |
| Prefix | 10.0.1.0/24 |
| Status | Active (required — not Container) |
| Tag | eda-fabric-dc2-systemip |
| Allocation CR | nb-fabric-dc2-systemip, spec.type: ip-address |
| EDA pool CRD | IPAllocationPool |

#### 6.4.4 Management IP (type: ip-in-subnet)

| Item | Lab value |
| --- | --- |
| NetBox UI | IPAM → Prefixes → Add |
| Prefix | 192.168.100.0/24 |
| Status | Active |
| Tag | eda-fabric-dc2-mgmt |
| Allocation CR | nb-fabric-dc2-mgmt, spec.type: ip-in-subnet |
| EDA pool CRD | IPInSubnetAllocationPool |

#### 6.4.5 ISL subnet (type: subnet)

Point-to-point ISL links use /31 subnets. The Allocation CR sets spec.subnetLength: 31.

| Item | Lab value |
| --- | --- |
| NetBox UI | IPAM → Prefixes → Add |
| Prefix | 10.255.0.0/16 |
| Status | Container (required for subnet allocation) |
| Tag | eda-fabric-dc2-isl |
| Allocation CR | nb-fabric-dc2-isl, spec.type: subnet, spec.subnetLength: 31 |
| EDA pool CRD | SubnetAllocationPool |

#### 6.4.6 Verify all pools

kubectl get allocation -n fabric-dc2
kubectl get allocation nb-fabric-dc2-vlan-pool -n fabric-dc2 -o jsonpath='matched={.status.matchedPrefixes}{"\n"}'
kubectl get indexallocationpools,ipallocationpools,subnetallocationpools,ipinsubnetallocationpools -n fabric-dc2

| Allocation CR | EDA pool kind | Pass |
| --- | --- | --- |
| nb-fabric-dc2-vlan-pool | IndexAllocationPool | status shows matched VLAN group |
| nb-fabric-dc2-asn-pool | IndexAllocationPool | matched ASN range |
| nb-fabric-dc2-systemip | IPAllocationPool | matched Active prefix |
| nb-fabric-dc2-mgmt | IPInSubnetAllocationPool | matched Active prefix |
| nb-fabric-dc2-isl | SubnetAllocationPool | matched Container prefix |

#### 6.4.7 Mode A — clab-3-tier-leaf-spine-dcgw (six Allocation CRs)

File: manifests/allocations-clab-3-tier-leaf-spine-dcgw.yaml. Seed NetBox IPAM with scripts/nb-test-allocation-pools-clab3tier.py (or nb-add-clab3tier-isl-ipv6-only.py for IPv6 ISL only). Full walkthrough:

| Allocation CR | type | NetBox tag | Prefix / object | subnetLength |
| --- | --- | --- | --- | --- |
| nb-clab3tier-systemip | ip-address | eda-clab3tier-systemip | 10.0.10.0/24 Active | — |
| nb-clab3tier-mgmt | ip-in-subnet | eda-clab3tier-mgmt | 192.168.110.0/24 Active | — |
| nb-clab3tier-isl | subnet | eda-clab3tier-isl | 10.254.0.0/16 Container | 31 |
| eda-isl-ipv6 | subnet | eda-clab3tier-isl-ipv6 | 2005::/64 Container | 127 |
| nb-clab3tier-asn-pool | asn | eda-clab3tier-asn | ASN range | — |
| nb-clab3tier-vlan-pool | vlan | eda-clab3tier-vlan | VLAN group | — |

## 7. Decoupling topology from allocations

### 7.1 Two paths, one namespace

EDA namespace fabric-dc2 — two independent paths:

| Path | Trigger | EDA objects | sync.enabled |
| --- | --- | --- | --- |
| Topology (DCIM) | ApplyTopology | TopoNode, Interface, TopoLink | false (no EDA→NetBox device push) |
| Allocations (IPAM) | Allocation CRs | IPAllocationPool, SubnetAllocationPool, IndexAllocationPool | Independent of sync |

| NetBox source (Mode B) | Consumed by |
| --- | --- |
| DCIM — Site, Devices, Cables (your design) | ApplyTopology |
| IPAM — Prefixes, VLAN Groups, ASN Ranges (tagged pools) | Allocation CRs |

| Dimension | Topology (DCIM) | Allocations (IPAM) |
| --- | --- | --- |
| Scoped by | NetBox site (ApplyTopology.spec.siteName) | Tags on IPAM objects |
| Site association | Required for devices/cables | Not required |
| EDA workflow | ApplyTopology | Allocation + ApplyAllocation |
| Controlled by sync.enabled | Yes (EDA→NetBox push) | No |
| Import direction | NetBox → EDA (when managed) | NetBox pool defs → EDA pools |
| Export direction | EDA → NetBox (when sync on) | EDA allocations → NetBox IPAM objects |
| EDAManaged on source | Mode A: all synced DCIM | Never on pool definitions |
| EDAManaged on consumption | N/A | Individual allocated IPs/VLANs/ASNs |

### 7.1.1 Bootstrap allocation pools vs NetBox-backed pools

Two different mechanisms can create SubnetAllocationPool (and other pool kinds) in an EDA namespace:

| Source | How it appears | Allocation CR (netbox.eda.nokia.com)? | Prefix / range in NetBox? |
| --- | --- | --- | --- |
| Namespace bootstrap | Copied from template namespace (eda.nokia.com/bootstrap: "true") | No | No |
| NetBox app | Allocation reconciles tagged IPAM → pool | Yes | Yes (parent pool object) |

Webhooks do not create pool definitions; they refresh reconciliation after IPAM changes. The Netbox Allocation CR is the integration object for NetBox-backed pools.

Lab migration (e.g. IPv6 ISL eda-isl-ipv6):

- Create NetBox container prefix (2005::/64, tag eda-clab3tier-isl-ipv6).

- Apply Allocation eda-isl-ipv6 with subnetLength: 127.

- Confirm status.matchedPrefixes on the Allocation.

- Only then remove a bootstrap-only SubnetAllocationPool with the same name if it never links to NetBox (avoid deleting a pool already owned by a matched Allocation).

### 7.2 Combined deployment patterns

#### Pattern 1 — Clab / EDA-managed fabric (lab default)

| Component | Setting |
| --- | --- |
| Namespace | clab-srl-leaf-spine-dcgw |
| Instance.sync.enabled | true |
| Topology | Born in EDA; mirrored to NetBox |
| Allocations | Optional Allocation CRs for VLAN/ASN/IP pools |
| NetBox DCIM | Read-only mirror (EDAManaged) |
| NetBox IPAM | Tagged pools + EDAManaged on consumed values |

#### Pattern 2 — Greenfield planned fabric (NetBox design source)

| Component | Setting |
| --- | --- |
| Namespace | fabric-dc2 (dedicated) |
| Instance.sync.enabled | false |
| Topology | Modelled in NetBox → ApplyTopology |
| Allocations | Allocation CRs + IPAM pools (can pre-exist before devices) |
| NetBox DCIM | Your source of truth (no EDAManaged) |
| NetBox IPAM | Pools untagged/EDAManaged only on allocations |

#### Pattern 3 — Hybrid (common in production)

- NetBox-managed planned site imported to EDA (sync.enabled: false)

- Shared IPAM pools tagged per namespace (e.g. eda-fabric-dc2-vlan vs eda-clab-vlan)

- Same NetBox server, separate Instance + webhook per namespace

- Fabric / Init / NodeProfile reference allocation pools in their namespace

### 7.3 What is shared vs isolated

| Shared across namespaces (lab) | Per namespace (always) |
| --- | --- |
| NetBox API token value (one token in NetBox UI, same literal in each netbox-api-token secret) | K8s Secret objects (netbox-api-token, netbox-webhook-signature) |
| NetBox server URL (Instance.spec.url) | Webhook URL path (.../webhook/<namespace>/netbox) |
|  | Webhook signatureKey (unique signing secret) |
|  | Instance CR, sync.region / sync.tenant |

### 7.4 Recommended build order

- Create EDA namespace via namespace instantiation — edactl namespace bootstrap create or EDA Namespace CR (NodeProfile, NodeUser, Init, pools)

- Secrets + Instance + NetBox webhook for this namespace

- Allocation CRs + NetBox IPAM pools (optional but do before Fabric/onboarding)

- NetBox DCIM: site, devices, interfaces, cables

- ApplyTopology

- ApplyAllocation if webhooks did not reconcile pools

## 8. Multi-namespace conventions and permissions

### 8.1 Webhook URLs and tag naming

One webhook URL per namespace — the path includes namespace and Instance name:

Examples (webhook — use address reachable from NetBox pod, not browser localhost):

https://<EDA-WEBHOOK-HOST>:9443/core/httpproxy/v1/netbox/webhook/clab-3-tier-leaf-spine-dcgw/netbox
https://<EDA-WEBHOOK-HOST>:9443/core/httpproxy/v1/netbox/webhook/clab-srl-leaf-spine-dcgw/netbox
https://<EDA-WEBHOOK-HOST>:9443/core/httpproxy/v1/netbox/webhook/fabric-dc2/netbox

| Access from | EDA URL |
| --- | --- |
| Your browser (UI, EDA Explorer) | https://localhost:9443 |
| NetBox webhook (HTTP call from NetBox pod) | https://eda-api.eda-system.svc.cluster.local:443/... or LB IP from kubectl get svc eda-api -n eda-system |

Suggested tag naming to avoid cross-namespace pool collisions (fabric-dc2 lab):

| Namespace | VLAN | ASN | System IP | Mgmt IP | ISL IPv4 | ISL IPv6 |
| --- | --- | --- | --- | --- | --- | --- |
| clab-3-tier-leaf-spine-dcgw | eda-clab3tier-vlan | eda-clab3tier-asn | eda-clab3tier-systemip | eda-clab3tier-mgmt | eda-clab3tier-isl → 10.254.0.0/16 Container, /31 | eda-clab3tier-isl-ipv6 → 2005::/64 Container, /127; pool eda-isl-ipv6 |
| clab-srl-leaf-spine-dcgw | eda-clab-vlan | eda-clab-asn | eda-clab-systemip | eda-clab-mgmt | eda-clab-isl | — |
| fabric-dc2 | eda-fabric-dc2-vlan | eda-fabric-dc2-asn | eda-fabric-dc2-systemip | eda-fabric-dc2-mgmt | eda-fabric-dc2-isl | — |

Device tags remain key=value (e.g. eda.nokia.com/node-profile=fabric-dc2-srlinux-26.3.1).

### 8.2 API token and webhook secrets (lab convention)

This lab uses one NetBox API token across all EDA namespaces. Create a single token in NetBox (Admin → API tokens) with the permissions. Store the same literal value in each fabric namespace’s Secret/netbox-api-token. Each namespace still has its own Secret object — EDA only reads secrets in the Instance’s namespace.

Webhook signing secrets are always per namespace. Each fabric gets a unique signatureKey in Secret/netbox-webhook-signature and a matching NetBox webhook whose URL path includes that namespace. Do not reuse webhook secrets across namespaces.

| Credential | This lab | Alternative (stricter isolation) |
| --- | --- | --- |
| NetBox API token | One token; duplicate value into each namespace’s netbox-api-token secret | Issue a separate NetBox token per fabric with scoped object permissions |
| Webhook signatureKey | Unique per namespace + dedicated NetBox webhook | Same — always per namespace |

You can mix approaches: e.g. shared API token for all Mode A clab fabrics, but a dedicated token for a production fabric-dc2 namespace with tighter DCIM scope.

### 8.3 NetBox API permissions

These are the object permissions on the NetBox API token referenced by Instance.spec.apiToken. EDA calls the NetBox REST API as that token user.

| Mode | NetBox token needs | Why |
| --- | --- | --- |
| NetBox-managed import only (sync.enabled: false) | IPAM: read+write (for allocations); DCIM: read | ApplyTopology workflow reads site/devices/cables; it does not write DCIM back |
| EDA-managed sync (sync.enabled: true) | IPAM: read+write; DCIM: create/update/delete | Sync creates and updates Site, Device, Cable, interfaces in NetBox |
| Allocations only | IPAM: read+write; DCIM: read | Pools and consumed values only |

Also required: Extras > Tag, Extras > Custom Field (for EDAManaged / eda_managed automation).

- write); webhook + event rule per namespace (§5.3 for Mode B); ENFORCE_GLOBAL_UNIQUE=false

- EDA: namespace + namespace instantiation (§5.2 / §3.2)

- EDA: secrets + Instance CR → verify status.reachable: true (§4.3 Mode A, §5.4 Mode B)

- EDA: Allocation CRs (pool tags must already exist on NetBox IPAM)

- NetBox: DCIM site/devices/interfaces/cables (Mode B only — §5.5)

- EDA: ApplyTopology (§5.6, Mode B) or let sync run (Mode A)

- EDA: ApplyAllocation if pools did not reconcile via webhook

# Part IV — Operations and Constraints

## 9. EDA transactions

Individual CR changes use EDA transactions (atomic, Git-backed). Distinct from ApplyTopology workflow ops (create / reconcile / replace).

### 9.1 Per-resource operation types

| Op | Resource exists | Resource missing | Behaviour |
| --- | --- | --- | --- |
| create | Fails | Creates | Strict create only |
| modify | Updates | Fails | In-place update |
| replace | Full overwrite | Creates (upsert) | Whole resource replaced |
| patch | Field update | Fails | JSON Patch (RFC 6902) |
| delete | Deletes | Fails | Remove by GVK + name + namespace |

### 9.2 Rule of thumb (NetBox-imported fabric)

| Goal | Use |
| --- | --- |
| Patch TopoNode label or npp.mode | modify or patch |
| Redefine whole TopoNode | replace |
| Add TopoLink without reconcile | transaction create or kubectl apply |
| Add device from NetBox | ApplyTopology with Create workflow |
| Remove one resource | delete |

### 9.3 Transaction vs topology import

| Mechanism | Orphans in namespace |
| --- | --- |
| Transaction ops on named CRs | Untouched |
| ApplyTopology reconcile | Deleted if not in site spec |
| NetworkTopology operation: create | Untouched |

### 9.4 Execution and inspection

- dryRun: validate without pushing to nodes

- detailLevel: basic | standard | detailed

- CLI: edactl apply|replace|patch|delete -f ... -d -m "message"

- Inspect: edactl transaction <id> cr-change|node-change

See EDA transactions documentation for full API and rollback (Revert / Restore).

## 10. Constraints and anti-patterns

| Mistake | Consequence |
| --- | --- |
| One namespace, clab + NetBox planned | ApplyTopology Reconcile removes TopoNodes not in the NetBox site — live fabric nodes in that namespace are deleted |
| sync.enabled: true on NetBox-design namespace | EDA overwrites your DCIM |
| ApplyTopology on EDAManaged site | Conflict / workflow failure |
| Same IPAM tag on pools for two namespaces | Wrong pool matched / shared consumption |
| Stale API token secret | Empty Allocation.status; pools never appear |
| ENFORCE_GLOBAL_UNIQUE=true with overlapping topologies | Duplicate IP allocation failures |

## References

This document is a summary of the full NetBox-EDA technical documentation. Both documents, together with the scripts and manifests they reference, are published in the integration repository below.

Repository: https://github.com/dtrichards01/netbox-eda-integration

Full guide: docs/NetBox-EDA-Technical-Documentation.md — full technical documentation, adding the CR reference, the implementation procedures, the manual test guide and the appendices.

This summary: docs/NetBox-EDA-Technical-Documentation-public-v1.docx — this document.

Scripts: scripts/ — seeding, verification and test scripts referenced throughout.

Manifests: manifests/ — Instance, ApplyTopology and Allocation CR examples, plus secret templates.

EDA docs: https://docs.eda.dev/ — Nokia Event-Driven Automation documentation.

EDA transactions: https://docs.eda.dev/latest/user-guide/transactions/ — transaction API, Revert and Restore.

NetBox: https://netboxlabs.com/ — NetBox documentation and releases.

Review feedback: Please raise comments as issues in the repository above, or return annotated copies of this document.